Hollyhock Single Pass Review – Landscape Guidelines and Designer’s Checklist

March 28, 2018 Edition (One Page)

______Plan is required to be submitted with the Construction Drawings, or no later than the Dry-in review.

______A plan for an operable irrigation system or approved water collection system is required.

______Landscape selection and arrangement reflects a high degree of planning.

______There are not a large number of a single variety of a cultivar present.

______Plants are varied in size and color.

______A lot with a back yard adjoining another yard, neither of which is on a preserve, shall provide a minimum of

two six foot tall evergreen trees at the lot line, but not planted where the mature tree will extend over the lot line.

______Grass sod is placed on all exposed disturbed areas on the lot.

______Evergreen foundation shrubs, a minimum of 18” tall are placed fully around the foundation, 36” o.c.

______A second layer of 8” tall min. evergreen shrubs are provided on the front and visible sides, between shrubs.

______More extensive planting is required on a corner lot. Evergreen trees should be considered at corners.

______No simple ringing the home with plants with no consideration or design and aesthetics.

______Three medium-growing hardwood shade trees (two in phase three) are planted on the lot of at least 2.5"

caliper (8-10' overall height), two placed in the front yard. Healthy hardwood trees that remain may be

credited. The three required trees are of different species. Two 10’ tall palms may replace one hardwood.

______HVAC equipment is screened with taller evergreen shrubs or trees that will fully conceal the equipment

within a two year period. Minimum initial planting height matches the equipment height.

______There are no large areas of mulch without shrub plantings.

(Mulch in beds is suggested to retain moisture, limit weeds and for a neat well kept appearance).

______Outside play or sports equipment are screened from neighbors and street and is constructed, painted or

stained in colors that blend with nature. No primary colors permitted over 6 ' in height.

______Planting of fruit and vegetable plants and trees in and around the landscape and in areas of the yard are

encouraged and may be substituted for required evergreen shrubs and hardwoods on a case by case basis.

______Pine straw is not used around a foundation or within wetlands due to its potential for sustaining fire

______Landscaping as approved by the ARC is installed prior to home occupancy, unless occupancy prior to

landscaping completion has been approved by the ARC for a good cause. If an exception is granted, then

occupancy is no earlier than 30 days from landscaping completion.

_____ Homes within 30 feet of a wetlands/preserve area may clear scrub brush within the wetlands for a distance

of 30 feet from the home. No disturbance of trees larger than 2” caliper. Also, no planting of grass in these areas.

______No work is started on the landscape work until the plan has been approved by the Hollyhock ARC .

_____ At the edge of the wetlands, an approved sign stating “Protected Natural Area” must be installed.

_____ Provide taller evergreen shrubs on the side of the propane tank to screen it from neighbor’s views.

The landscape plan must be provided as a scaled drawing with the home, accessory buildings, decks or patios

shown. Plants are required to be shown at their mature scale. A schedule of all vegetation to be added must be

provided on the same drawing.

Provide the name of the landscape contractor, their Horry County License number and their insurance

endorsements if they are not under the supervisory control of the General Contractor.

PROVIDE THIS FORM WITH YOUR LANDSCAPE SUBMITTAL