Hollyhock Single Pass Review – Architectural Guidelines and Designer’s Checklist

March 28, 2018 Edition (2 pages)

Hollyhock designs are predicated on custom homes with a Coastal Cottage theme. This includes Craftsman, Bungalow, Nantucket and the indigenous styles of the American East Coast. Cottage designs in the Dan Sater book “Cottage and Villa Home Plans” are often held out as the standard. However, Hollyhock has benefited from the talents of several custom home designers. A list of these individuals is available from the HOA.

Submit this checklist with your plans. Check if accepted; N/A for not applicable; X for not met (explain separately)

SITE DESIGN

_____Elevations exposed to Carolina Forest Blvd (1-23; 356-363 and 385) require a high level of aesthetics.

_____Home is within setback requirements and the front face is similar distance to the street curb as adjacent home.

Setbacks from property line: front - 20’; side - 10’; side at street - 15’; rear - 15’; lots on preserve or common area – 30’.

_____Home on lake or common area does not block 125 degree field of view of adjacent (or future) home.

Future home is projected with the same rear plane as your design where your home is centered left and right on your lot.

_____ Any proposed swimming pool, deck or gazebo are fully detailed on the initial plan submittal.

_____ Drainage of rain water is either directed to the street or to the rear, but not sideways onto neighboring lots.

_____ A landscape plan is required. See separate attachment.

OVERALL MASSING, EXTERIOR FINISHES

_____The overall mass of the home has good proportions and a complementary roof line. A single roof does not cover

the majority of the home without the use of cross gables or dormers to provide a scale to the roof.

_____Floor plans and elevations are coordinated so that long blank featureless walls do not occur on any elevation.

Provide a complete architectural wall section from foundation through a window and through roof. Include frieze.

_____No more than 75% of a two story wall is comprised of a single material. Exclude foundations and windows in

calculation. Approved exterior wall materials include cement products in traditional horizontal or vertical siding patterns

such as beveled, ship lapped, tongue and groove, board and batten, or shake. Conventional three coat Portland Cement

stucco is allowed where appropriate to the style of the home. EIFS, vinyl siding and Masonite are not approved.

_____ Materials are consistent with coastal cottage aesthetic in type and color and balanced on all sides of the home.

_____Porch rails are required but may be eliminated as described under Porches below.

_____Residence has a minimum of 2,400 SF of enclosed heated living area (as measured by real estate standards)

Exception: Lots 820 through 847 and lots 852 through 904: Heated living area is 1800 SF, 1200 SF main level.

Measurement excludes porches, decks, garage, or other unheated spaces.

_____Residence does not exceed a maximum height of 35' above finished grade, measured by Horry County standards.

_____Finished main floor is not less than 24 inches above the finished adjacent grade on any side of the home.

_____No one level porch, two stories high. In multi-story homes, ceiling height is at least 9' on the main level and 8' on

the other levels. In a single story home, there is a minimum of 10' ceilings throughout, except storage, closets and baths.

_____Foundation materials are brick, stone or tabby stucco using #3 shell. One material is used fully around the home.

ROOF

_____ No single roof material exceeds 75% of the total roof, except when roof is metal. Adjustments may be approved

based on roof lines. Porch roofs, pediments and dormers must have standing seam metal roof, in an approved color.

_____ Roof pitch minimum is 6:12 except where metal roof is used.

_____ Shingles must be architectural, minimum 240# per square. (Minimum bundle weight 80#) No three tab.

_____ Gutters are along all eave lines and complement trim color. Discharge is minimum 8’ from side property line.

_____ Vents and exhaust piping are on rear of home and painted to match roof color.

_____ Solar collectors should be placed flush to roof plane.

_____ Lightening protection cables should be placed so not exposed to street view in as much as possible.

_____ Fireplace exhaust stacks should be enclosed in materials same as the home. Conceal spark arrestors with shroud.

WINDOWS, DOORS and SHUTTERS

_____ Front entry door is prominent and inviting, having high level of detail and wide casing around doorway.

_____ Windows align vertically with windows and dormers above. Paired windows have 4” mullion separation.

_____ Windows in front and sides of home shall have 5/8” min. grills on upper sash as a minimum.

_____ Windows shall be fully trimmed on all sides of the home. Minimum 4” casing.

_____ Windows may be aluminum or vinyl clad. Full vinyl windows must be: Simonton-Master Pro; Jeld-Wen Select

Vinyl; Ply-Gem Pro Series Classic; Windsor Legend or HBR; Pella 350; Viwinco – S; or others if ASTM E1996/E1886.

_____ Single hung windows are approved. Full screens must be used on the front of all homes if screens are used.

_____ Large picture windows and burglar bars are not approved. Glass blocks are not approved where street facing.

_____ Designs for hurricane shutters continue to evolve. Currently approved are impact resistant operable shutters

and roller shutters in fully concealed header with integral side trim. Systems with exposed wall studs are not approved.

The ARC will consider approval of concealed hurricane shutters when provided full head, jamb and sill details.

_____ If window shutters are used, they should match window width and applied to all windows on front and sides.

_____ No windows can be blacked out nor have signage, nor have non-neutral shades or curtains exposed.

GARAGES, PORCHES, DECKS

_____ Two car garage minimum. Three car maximum on single lot. Double lot may have two-double garage doors.

_____ Detached garages are permitted when connected with breezeway or porte-cochere and match home’s design.

_____ Garage doors should be carriage-house in style, complement home color, have windows when forward facing.

_____ Porches are substantial in appearance and match the architecture of the home. Screens shall be dark.

_____ Porches have 10” columns in all first floor locations. 8” columns minimum second level.

_____ Porch railing shall be milled decay resistant material or approved synthetic material, or approved metal design.

Railing may be eliminated where not required by code and 10” column is placed over 16” base which is 48” H; or

where decoratively trimmed 8” columns are in pairs at each required column location.

_____ Railing should be painted to match trim. Approval of color is required.

_____ Porch foundation should be same as home. Brick may be Flemish bond with header removed (leaving gaps).

FENCES AND SCREEN WALLS

_____ Where placed on lake lots, black aluminum fence, flat rail top, stiles 4” o.c. min. (pup fence ok) 5 feet tall only.

_____ Fences may come to the back plane of the home or to side garage door, where 15 feet back from front plane.

_____ Vinyl fence may be used in other locations. Meet 75MPH wind load; 5”x5” posts 8 feet o.c.; routed rails; 5 foot

height only, except one foot lattice may be placed atop a five foot tall solid panel fence. White finish only.

_____ Masonry fences may be used. The design details and location shall be submitted for review. Masonry enclosures

are required for pool equipment and emergency generators for noise control.

_____ Where portable gas tanks (max. 25 gallon allowable) are expected to be added to the home, a screen wall between

the home and the street is required at a height to cover the tank and valves. (5 foot tall recommended). Larger tanks

are required to be buried. If a buried gas tank is not part of the construction, and gas is piped, a screen wall is required.

_____ Place fence within 4 inches of lot lines or 5 feet off lot lines. Neighbors may utilize a portion of your fence.

DRIVEWAYS and MAILBOXES

_____ Driveways shall be broom finished or tabby concrete, brick pavers, epoxy or stamped concrete. Strips may

incorporate stone. Asphalt is not approved. Width at street shall not exceed 18 feet total unless circular drive is provided.

_____ A 15” RCP culvert is required at drainage swales. Sidewalks shall take precedence over driveways.

_____ No markers, signs or reflectors are permitted at the end of the driveway. Integral graphics may be approved.

_____ Mailbox shall be placed to the inside of the drive. Use Carolina Mailbox, Imperial 310r-6. 3” high numbers.

OTHER ITEMS

_____ A place for trash containers should be provided on the outside of each home. A self contained storage unit or

screened area will be required with containers placed on a concrete pad.

_____ Antennas for short wave or television and satellite dishes shall be screened. Roof installation requires approval.

_____ Audible alarms for security shall be screened. Flashing lights for emergency use only.

_____ Exterior lighting shall complement the home. No colored lights, except seasonal. No light directed to neighbor.

_____ Flagpole 6 feet long with one US Flag, 4” x 6” maximum may be placed on your home. No flagpoles in the yard